[image: image1.jpg]LEYU! ©

S



    

          乐清市乐宇自动化电气有限公司
                Yueqing Leyu Electric Automation Co., Ltd.

150W Waterproof Single Output Switching Power Supply    LPV-150 series
Features:
Constant voltage design
Universal AC input/Full range
Withstand 300VAC surge input for 5 seconds
Protections: short circuit/ over load/ over voltage
Cooling by free air convection
Small and compact size
Fully encapsulated with IP67 level
Fully isolated plastic case
Suitable for LED lighting and moving sign applications
100% full load burn-in test
Low cost, high reliability
2 year warranty
	SPECIFICATION
	

	OUTPUT
	

	Model
	LPV-150-12
	LPV-150-15
	LPV-150-24
	LPV-150-36
	LPV-150-48

	DC Voltage
	12V
	15V
	24V
	36V
	48V

	Rated Current
	10A
	8A
	6.3A
	4.2A
	3.2A

	Current Range
	0-10A
	0-8A
	0-6.3A
	0-4.2A
	0-3.2A

	Rated Power
	120W
	120W
	151.2W
	151.2W
	153.6W

	Ripple&Noise
	200mVp-p
	200mVp-p
	200mVp-p
	200mVp-p
	200mVp-p

	Voltage Tolerance
	±5%

	Line Regulation
	±1%

	Load Regulation
	±2%

	Setup,Rise Time
	2000ms, 25ms/230VAC2000ms, 25ms/115VAC

	Hold Up Time
	50ms/230VAC   14ms/115VAC at full load

	
	

	INPUT
	

	Voltage Range
	180-305VAC47-63Hz;254~431VDC

	AC Current
	2.2A/115V1.2A/230V

	Efficiency
	87%
	88%
	98%
	89%
	90%

	Inrush Current
	Cold Start60A at 230VAC

	Leakage Current
	0.25mA/240VAC

	
	

	PROTECTION
	

	Over Load
	110-150% rated output power

	
	Protectiontype: Hiccup mode, recovers automatically after fault condition is removed

	Over Voltage
	13.5-18V
	17-25V
	27~35V
	40-49V
	52-63V

	
	Protection type: Shut down o/p voltage, re-power on to recover

	
	

	ENVIRONMENT
	

	Working Temp.Humidity
	-25 ~ +70ºC (Refer to "Derating Curve") 20-90% RH non-condensing

	Storage Temp.Humidity
	-40ºC~+80ºC; 10%~95%RH

	Temp.Coefficient
	±0.03%/ºC(0~50ºC)

	Vibration
	10~500Hz,2G 10min,/1cycle,period for 60min,each along X,Y,Z axes

	
	

	SAFETY
	

	Withstand Voltage
	I/P-O/P:3KVAC

	Isolation Resistance
	I/P-O/P>100M Ohms / 500VDC / 25ºC / 70% RH

	
	

	STANDARD
	

	Safety Standard
	UL8750, CSA C22.2 No.250.13-12, UL879,CSA C22.2 No.207-M89, IP67 approved, design refer to EN60950-1

	EMCStandard
	EN61000-4-2,3,4,5,6,8,11,EN55024,light industry level, criteria A

	
	

	OTHERS
	

	Dimension
	191*37*37.5mm(L*W*H)

	Weight
	0.78kg;20pcs/13.6kg/0.55CUFT

	
	

	NOTE
	

	1. All parameters NOT specially mentioned are measured at 230VAC input, rated load and 25ºC of ambient temperature.

	2. Ripple & noise are measured at 20MHz of bandwidth by using a 12" twisted pair-wire terminated with a 0.1uf & 47uf parallel capacitor.

	3.Tolerance: includes set up tolerance, line regulation and load regulation.

	4. Derating may not be needed under low input voltage. Please check the static characteristics for more details.



