| OPIP-4000C |  |  |  |
| --- | --- | --- | --- |
| Pure Sine Wave Inverter |  |  |  |
| Rated Power: | 4000W | Frequency: | 50Hz / 60Hz |
| Peak Power: | 8000W | Unit Size: | 446*199*146mm |
| Waveform: | Pure Sine Wave | Unit Weight / G.W.: | 10100g / 12300g |
| DC Input Voltage: | 12V / 24V | Box Size: | 575*285*240mm |
| AC output Voltage: | 110V / 220V | Quantity/CTN: | 1PCS |

| Technical parameters |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | OPIUP-4K-1 | OPIUP-4K-1 | OPIUP-4K-1 | OPIUP-4K-2 | OPIUP-4K-2 | OPIUP-4K-2 |
| Rated power | 4000W |  |  |  |  |  |
| Peak power | 8000W |  |  |  |  |  |
| Input voltage | DC12V | DC24V | DC48V | DC12V | DC24V | DC48V |
| Output voltage | 100VAC or 110VAC or 120VAC±5% |  |  | 220VAC or 230VAC or 240VAC±5% |  |  |
| Unload current less than | 0.8A | 0.5A | 0.2A | 0.8A | 0.5A | 0.2A |
| Output frequency | 50Hz±0.5Hz or 60Hz±0.5Hz |  |  |  |  |  |
| Output waveform | Pure Sine Wave |  |  |  |  |  |
| Waveform distortion | THD<3%(Linear load) |  |  |  |  |  |
| USB port | 5V 1A |  |  |  |  |  |
| Max.efficiency | 90% |  |  |  |  |  |
| Input voltage range | 10-15.5V | 20-31V | 40-61V | 10-15.5V | 20-31V | 40-61V |
| Low voltage alarm | 10.5±0.5V | 21±0.5V | 42±1V | 10.5±0.5V | 21±0.5V | 42±1V |
| Low voltage protection | 10±0.5V | 20±0.5V | 40±15V | 10±0.5V | 20±0.5V | 40±15V |
| Over voltage protection | 15.5±0.5V | 31±0.5V | 61±1V | 15.5±0.5V | 31±0.5V | 61±1V |
| Low voltage recover | 13±0.5V | 24±0.5V | 48±1V | 13±0.5V | 24±0.5V | 48±1V |
| Over voltage recover | 14.8V±0.5V | 29.5V±0.5V | 59V±1V | 14.8V±0.5V | 29.5V±0.5V | 59V±1V |
| Protection function | Low voltage |  | Alarm at first, voltage continously reduce. LED Red light on & shut sown. |  |  |  |
|  | Over voltage |  | LED Red light on, shut down |  |  |  |
|  | Over load |  | LED Red light on, shut down |  |  |  |
|  | Over temperature |  | Alarm at first, tempurature continously rise.LED Red light on & shut sown |  |  |  |
|  | Short circuit |  | LED Red light on |  |  |  |
|  | Input reverse polarity |  |  | Fuse burn-out |  |  |
| Working temperature | -10°- +50° |  |  |  |  |  |
| Storage temperature | -30°- +70° |  |  |  |  |  |
| Dimension(mm) | 446×199×146mm |  |  |  |  |  |
| Packing(mm) | 575×285×240mm |  |  |  |  |  |
| Net./gross weight(g) | 10100/12300g |  |  |  |  |  |
| QTY/Ctn | 1 Pcs |  |  |  |  |  |
| Meas./Ctn(mm) | 575×285×240mm |  |  |  |  |  |
| Gross weight/Ctn(g) | 12300g |  |  |  |  |  |
| Warranty | 2 years |  |  |  |  |  |
| Configuration | Standard |  |  |  |  |  |
| Cooling method | Intelligent air cooling |  |  |  |  |  |
| Note | Our products update constantly.The technical parameter is for reference only.Please refer to our real product. |  |  |  |  |  |
| Wanjia Industrial Park, Beibaixiang Town, Yueqing City, Zhejiang Province, CHINA |  |  |  |  |  |  |