# ZSN3-5K0 智能工业电源

| Model |  |  | ZSN3-5K0-12 | ZSN3-5K0-24 | ZSN3-5K0-36 | ZSN3-5K0-48 | ZSN3-5K0-72 | ZSN3-5K0-96 | ZSN3-5K0-110 | ZSN3-5K0-150 | ZSN3-5K0-220 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OUTPUT |  | DC Voltage | 12V | 24V | 36V | 48V | 72V | 96V | 110V | 150V | 220V |
|  |  | Rated Current | 333A | 208A | 138.8A | 104A | 69A | 52A | 45A | 33A | 22.7A |
|  |  | Current Range | 0 - 333A | 0 - 208A | 0 - 138.8A | 0 - 104A | 0 - 69A | 0 - 52A | 0 - 45A | 0 - 33A | 0 - 22.7A |
|  |  | Rated Power | 4000W | 5000W |  |  |  |  |  |  |  |
|  |  | Ripple | 250mV | 270mV | 300mV | 400mV | 500mV | 600mV | 850mV | 900mV | 1000mV |
|  |  | Optimal Range of Constant Current | 6 - 12V | 12 - 24V | 18 - 36V | 24 - 48V | 36 - 72V | 48 - 96V | 55 - 110V | 75 - 150V | 110 - 220V |
|  |  | voltage accuracy | ± 1.0% |  |  |  |  |  |  |  |  |
|  |  | linear adjustment rate | ± 1.0% |  |  |  |  |  |  |  |  |
|  |  | load regulation | ± 1.0% |  |  |  |  |  |  |  |  |
|  |  | Setup, Rise, Hold Up Time | 1500mS，100mS/230VAC(at full load） |  |  |  |  |  |  |  |  |
| INPUT | Voltage Range |  | 180 - 264VAC / 245 - 370VDC |  |  |  |  |  |  |  |  |
|  | Frequency Range |  | 45Hz - 65Hz |  |  |  |  |  |  |  |  |
|  | power factor |  | PF≧0.6/230VAC(at full load) |  |  |  |  |  |  |  |  |
|  | Efficiency |  | 83 % | 86 % | 87 % | 89 % | 90 % | 90 % | 90 % | 91 % | 91 % |
|  | AC Current |  | ＜ 55A |  |  |  |  |  |  |  |  |
|  | Leakage Current |  | ＜ 3.0mA / 240VAC |  |  |  |  |  |  |  |  |
| PROTECTION | short circuit |  | Enter a constant current |  |  |  |  |  |  |  |  |
|  | Over Temp |  | The output is turned off, and it automatically resumes or restarts when the temperature drops |  |  |  |  |  |  |  |  |

| FUNCTION | output voltage adjustment | 0 - 13.2V | 0 - 26.4V | 0 - 39.6V | 0 - 52.8V | 0 - 79.2V | 0 - 105.6V | 0 - 121V | 0 - 165V | 0 - 242V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | constant current output adjustment | 0 - 333A | 0 - 208A | 0 - 138.8A | 0 - 104A | 0 - 69A | 0 - 52A | 0 - 45A | 0 - 33A | 0 - 22.7A |
|  | External potentiometer | External potentiometer control (voltage, current) |  |  |  |  |  |  |  |  |
|  | Analog voltage control | 0 - 5V / 0 - 10V Control (voltage, current) |  |  |  |  |  |  |  |  |
|  | Auxiliary power supply | 12V 0.5A |  |  |  |  |  |  |  |  |
|  | remote control switch | By default, it will be powered on and powered off at a high level （3V-12V） |  |  |  |  |  |  |  |  |
| ENVIRONMENT | Working Temp | -20 - +60℃ |  |  |  |  |  |  |  |  |
|  | Working Humidity | -20 - 90% RH No condensation |  |  |  |  |  |  |  |  |
|  | Storage Temp., Humidity | -40 - +85℃, 10 - 95% RH No condensation |  |  |  |  |  |  |  |  |
|  | vibration resistance | 10 - 500Hz, 2G 10 min/cycle, 60 minutes each in X, Y, and Z axes |  |  |  |  |  |  |  |  |
| SAFETY | Insulation resistance | Input-to-output:100Mhms/500VDC/25℃/70%RH |  |  |  |  |  |  |  |  |
|  | Withstand Voltage | I/P-O/P :1.2KVAC I/P-FG:1.2KVAC O/P-FG:0.5KVAC |  |  |  |  |  |  |  |  |
| OTHERS | Dimension | 356*216*80mm（L*W*H） |  |  |  |  |  |  |  |  |
|  | Weight | 6.5 KG |  |  |  |  |  |  |  |  |
| REMARK | 1. All parameters are measured at 230VAC input, rated load and 25°C unless otherwise specified. 2. RIPPLE AND NOISE VOLTAGES ARE MEASURED AT 20UMHz bandwidth oscilloscope with 0.1 μ and 47 μ capacitance at the end of a 12-inch twisted pair cable, measured at 20MHZ bandwidth. 3. Accuracy: including setting error, linear regulation and load regulation. 4. If the input voltage is low, the output needs to be derated, please refer to the static characteristic curve for details. 5. The start-up time is measured when the machine is cool-in, and frequent switching on and off may increase the start-up time. |  |  |  |  |  |  |  |  |  |